LINUX QUADCOPTER
FLIGHT-CONTROL DEVELOPMENT

Simulation status, demonstrated software, constraints, and a staged route to hardware testing

| Report details | Value |
| --- | --- |
| Project | Dual-prototype cooperative quadcopter concept; Linux simulation and flight-control groundwork |
| Prepared | 5 October 2026 |
| Evidence base | My current workspace code and run logs, my terminal output, and the attached build/design brief |
| Status boundary | I have demonstrated software simulations; I have not documented a physical airframe, flight controller, sensors, or flight tests in this workspace. |

# Executive summary

I am developing a Linux-based simulation and visualization sandbox to support my larger dual-quadcopter build objective. My workspace now contains CUDA-capable 3D flow experiments, a 2D backward-facing-step solver with JSON checks, a small single-axis pitch state-space demonstration, a CUDA notebook, and VS Code launch configurations. I verified that PyTorch 2.14.1+cu132 detects my GeForce GTX 1650 and ran the smoke-flow and Coanda scripts on CUDA. These runs demonstrate working compute and visualization workflows, but I have not yet implemented or validated a quadcopter attitude-stabilization controller.

My attached design brief describes two near-identical aircraft, each with a deterministic H743-class flight controller, IMU, four-motor/4-in-1 ESC powertrain, ELRS pilot link, and a Raspberry Pi companion for autonomy, telemetry, logging, and inter-aircraft coordination. This is my target architecture; I have not established in the current project record that these parts have been acquired, wired, or flown.

My recommended engineering boundary I will keep high-rate stabilization, state estimation, motor mixing, arming, and immediate failsafes on the H743 flight controller. I will use Linux and the companion computer for development, simulation, logging, visualization, mission-level autonomy, and coordination. I will move from simulation to props-off bench and supported hardware-in-the-loop checks before any restrained or free-flight test.

# 1. Project objectives and scope

## 1.1 Current software objectives

Establish a reproducible Linux development environment that can run my CPU-capable Python work and CUDA-enabled PyTorch experiments.

Reconstruct and execute the original flow-visualization concepts: my smoke-flow test, curved-surface Coanda airflow demonstration, and backward-facing-step numerical/rendering exercise.

Make my results reviewable through GIF/PNG/PT outputs, JSON diagnostics, VS Code display support, a notebook runtime check, and documented launch commands.

Begin a separate, deliberately small pitch-axis dynamics example as a stepping stone toward my flight-control simulation, while keeping clear that a multirotor controller is not yet implemented.

## 1.2 System-level objective in the build brief

In my design brief, I propose two similar quadcopters that can fly independently and exchange state, sensor, mission, waypoint, health, and link information for cooperative tasks. I intend the duplicated platforms to support repeatable testing, fault isolation, and A/B comparison. I want each aircraft to remain locally controllable if its inter-aircraft link or companion computer fails; coordination is a higher-level function and must not be a dependency for basic attitude stability.

## 1.3 Intended vehicle architecture

| Subsystem in design brief | Intended responsibility | Evidence/status in this project |
| --- | --- | --- |
| H743-class flight controller | Real-time IMU acquisition, attitude/rate estimation, stabilization, motor mixing, arming and failsafes. | I describe this as the planned architecture; I have not demonstrated firmware, a board connection, or loop timing. |
| Four motors + 4-in-1 ESC; 6S target | Generate vehicle forces/torques from mixer outputs. | I list this as my target configuration; I have not supplied measured motor/ESC models or bench data. |
| IMU (gyro + accelerometer) | Core angular-rate and attitude sensing. | I plan to use this; I have not included sensor logs or calibration data. |
| Optical flow / LiDAR class sensor | Near-ground motion and altitude assistance. | I list this as an optional staged addition; I have no integration evidence yet. |
| ELRS receiver | Pilot commands, arming/mode selection, and manual/emergency override. | I plan to use this; I have not demonstrated receiver or failsafe tests. |
| Raspberry Pi 4/5 class companion | Linux autonomy, MAVLink, telemetry, logging, networking, and coordination. | I have an active Linux development host; I have not evidenced an embedded Pi installation. |
| Optional OAK-D Lite | Later visual perception experiments. | I list this as a future option only. |

# 2. Tools and software installed or used

| Tool or component | Observed use / configuration | Role |
| --- | --- | --- |
| Linux Mint Zena; Bash | My terminal shows Linux Mint repositories and a Bash shell. | My primary development host and command-line workflow. |
| NVIDIA driver stack | Driver 595.84; GTX 1650 Mobile (4 GB); I observed nvidia-smi reporting CUDA capability up to 13.2. | Provides host GPU access. I understand that driver CUDA reporting is not the same as a separately installed full CUDA toolkit. |
| nvidia-modprobe / kernel module | I installed nvidia-modprobe and ran `sudo nvidia-modprobe -u -c=0` and `sudo modprobe nvidia_uvm` to restore device nodes/compute access. | I resolved missing `/dev/nvidia*` access in my Linux setup. |
| Python virtual environment | I use the project `.venv` and activated it in my terminal. | Keeps my project dependencies separate from system Python. |
| PyTorch CUDA wheel | I observed `2.14.1+cu132`, `torch.version.cuda == 13.2`, CUDA available, and device name GTX 1650. | I use GPU tensor execution for the two 3D flow experiments. |
| NumPy / Matplotlib / ImageIO | Declared in requirements; used by numeric solver, plots, and animation generation. | Array calculations, plotting, and GIF output. |
| VS Code for Linux + Python tooling | I use `code .`; my repository includes `.vscode/launch.json` debug entries. | I edit/debug, launch scripts, and view generated JSON/image output. |
| Jupyter notebook | `cuda_smoke_test.ipynb` checks device availability and performs a small tensor operation. | Interactive confirmation of Python/PyTorch runtime. |

Environment-specific caveat I demonstrated CUDA successfully in my Linux terminal. The managed coding-host process may not have the same NVIDIA device nodes or permissions, so I use the terminal and interpreter that produced my successful probe when checking the local machine.

# 3. Python components implemented

| File | Implementation and outputs | Engineering interpretation |
| --- | --- | --- |
| `torchFileTest.py` | I implemented a 3D PyTorch smoke field with semi-Lagrangian advection, pressure projection, vorticity confinement, and density/velocity visualization. It writes `smoke.gif`, `smoke_3d_plane.gif`, `smoke_slice.png`, `smoke_slice.pt`, and telemetry JSON. | I use it as a GPU-capable flow visualization and projection diagnostic, not as a rotor/airframe model. |
| `coandaFlowTest.py` | I implemented a 3D injected jet/tracer over a curved surface, near-wall attachment/alignment force, and projection. It writes side and curved-surface GIFs, mid-slice PNG, and JSON telemetry. | I use it as a qualitative Coanda-style flow experiment, not as a validated aerodynamic model for my vehicle. |
| `readable_backward_facing_step_solver-3.py` | I implemented a NumPy 2D backward-facing-step predictor/projection solver with pressure/velocity plots and JSON validation checks. | I use it as a numerical/plotting exercise with lightweight sanity checks; its values do not establish benchmark accuracy. |
| `rotor_pitch_axis_state_space.py` | I added a shared two-state linear pitch-axis model, state matrices, configuration defaults, and RK4 integrator. | I treat it as a simplified second-order educational model. It lacks measured vehicle parameters, motor dynamics, nonlinearities, and a controller implementation. |
| `helicopter_pitch_phase_portrait_aggressive_damping.py` | I run eigenvalue, free-response, and phase-portrait calculations for an aggressively damped single-axis pitch model; it writes a PNG and can open it in VS Code. | I use this to illustrate how damping changes a linear response; the helicopter label/origin does not make it a quadrotor control validation. |
| `cuda_smoke_test.ipynb` | I use this notebook to report PyTorch device selection and run a small tensor operation. | I use it as a runtime/device check only, not as a flow or flight-control model. |

## 3.1 Pitch-axis model assumptions

My recovered model defaults are natural frequency 1.8 rad/s, cyclic gain 2.6, initial pitch 10 degrees, initial pitch rate zero, a 5-degree input step at 1 s, 10 s duration, and a 0.01 s RK4 step. The model state is [pitch angle, pitch rate]. In my aggressive portrait, damping ratio ζ = 1.0 gives repeated real eigenvalues near −1.8 s⁻¹; the recorded response falls below 0.5° by about 2.64 s. I treat these transcript-derived parameters as illustrative, not as identified dynamics for my target quadcopter.

# 4. Bash workflow and VS Code launch configuration

I have not added a standalone `.sh` script to the repository. My demonstrated Bash workflow consists of terminal commands and shell command sequences. I have saved VS Code launch configurations in `.vscode/launch.json`; these are debugger configurations, not Bash scripts.

## 4.1 Create and prepare the Python environment

cd ~/Documents/Codex/2026-10-05/referenced-chatgpt-conversation-this-is-an
python -m venv .venv
source .venv/bin/activate
python -m pip install --upgrade pip

For CPU execution, I install my declared project packages. For CUDA, I first install a PyTorch build compatible with my host driver using the official selector, then install the remaining requirements and verify the active interpreter.

python -m pip install -r requirements.txt
python -c 'import torch; print(torch.__version__, torch.version.cuda); print(torch.cuda.is_available()); print(torch.cuda.get_device_name(0) if torch.cuda.is_available() else "GPU not detected")'

## 4.2 Host GPU device-access recovery used

I initially had a device-access issue: the driver could identify the GPU, but my environment lacked usable NVIDIA device nodes/UVM access. I installed the missing helper and loaded the relevant module:

sudo apt update
sudo apt install nvidia-modprobe
sudo nvidia-modprobe -u -c=0
sudo modprobe nvidia_uvm
nvidia-smi

These are host setup commands that may require administrator access; I only need to repeat them if the device-access problem returns. Once the host device works, I use the CUDA-enabled PyTorch wheel in my active virtual environment. My successful probe printed `2.14.1+cu132`, CUDA `13.2`, `True`, and `NVIDIA GeForce GTX 1650`.

## 4.3 Demonstrated simulation commands

python torchFileTest.py --steps 4 --no-vscode --out-dir work/smoke-cuda-check
python coandaFlowTest.py --warmup-steps 0 --record-steps 4 --no-vscode --out-dir work/coanda-cuda-check

python readable_backward_facing_step_solver-3.py --steps 50 --pressure-iterations 20 --output work/bfs_quick.png --report work/bfs_quick_report.json
python helicopter_pitch_phase_portrait_aggressive_damping.py

I use `--no-vscode` to keep short batch runs noninteractive. By default, my output-viewing helpers can invoke `code -r <path>` when available. I open the project with `code .` and select one of my five saved debug configurations: quick smoke, quick Coanda, quick/full BFS, or pitch portrait.

## 4.4 Automation gaps worth closing

I will add a small `scripts/` directory with explicit setup/probe/run scripts (for example `check_cuda.sh`, `run_flow_demos.sh`, and `run_validation.sh`) once my CLI options are stable.

I will have each script use `set -euo pipefail`, check or resolve the project Python executable, create output directories, and print the interpreter/device before running.

I will keep host-driver repair commands separate from routine run scripts because they require administrator privileges and change host configuration.

I will pin or lock a known-good set of Python dependencies for reproducible runs and record OS, Python, PyTorch wheel, driver, GPU, and command in each run manifest.

# 5. Demonstrated execution and results

| Run | Observed execution | Output / interpretation |
| --- | --- | --- |
| Smoke flow, quick CUDA | I ran 4 steps with CUDA in 1.110 s. Final density 0–3.200, max velocity magnitude 0.479. At step 3 mean divergence 0.000101→0.000084, max 0.151678→0.092296; CFL 0.024. | I generated a smoke GIF, 3D plane GIF, PT slice, and PNG slice. This demonstrates my CUDA path and artifact generation. |
| Coanda, quick CUDA | I ran 4 recorded steps with no warmup on CUDA in 1.659 s. Tracer 0–0.862, max velocity 3.920. At final sample mean divergence 0.014428→0.008993; attached-tracer metric 0.698; CFL 0.176. | I generated side/curved GIFs and a mid-slice PNG, demonstrating my run path and telemetry fields. |
| Smoke flow, full captured run | I ran 100 steps on CUDA in 8.874 s. Max density 10.0, max velocity 3.538. Final sampled mean divergence 0.000309→0.000232; CFL 0.177. | I saved smoke animations, a PT slice, and a PNG slice; my run log is in the workspace. |
| Coanda, full captured run | I ran 70 warmup + 130 recorded = 200 steps on CUDA in 36.531 s. Tracer max 0.855, max velocity 5.429. Final sample mean divergence 0.016444→0.016096; attached tracer 0.581; CFL 0.244. | I generated side and curved-surface animations and a mid-slice PNG. I treat the diagnostic trend as illustrative, not as a convergence/accuracy claim. |
| Backward-facing step quick validation | I ran 50 steps and 20 pressure iterations; my JSON report says PASS for 13 checks. Mean absolute divergence 0.0947; L2 divergence 75.29; max absolute divergence 21.12; recirculation fraction 0.594. Finite fields, zero solid velocity, and inlet u=1.0 checks pass. | I produced a non-blank 2160×810 render with pressure color variation. These checks validate expected code/output behavior only; the divergence magnitude shows this is not a precision reference solution. |
| Pitch portrait | I ran the illustrative ζ=1.0 linear case; my earlier run evidence records settling below 0.5° at 2.64 s. | I generated a phase portrait/image and eigenvalue demonstration; this is not a tested multirotor controller. |

How I interpret these results My flow scripts demonstrate that the CUDA stack is operational and produce inspectable outputs. Their divergence statistics describe those specific numerical experiments; they do not establish physically accurate airflow around my aircraft. Likewise, a plot settling under my prescribed linear model does not show that the real vehicle will be stable.

# 6. Constraints encountered and present readiness

| Constraint / gap | Effect on the work | Practical response |
| --- | --- | --- |
| NVIDIA device nodes/UVM initially unavailable | The driver could identify my GPU, but my Python process could not initially use it. | I installed/used `nvidia-modprobe`, loaded `nvidia_uvm`, checked `nvidia-smi`, then installed and verified CUDA PyTorch. |
| CPU-only PyTorch at an earlier point | My earlier interpreter reported a CPU build and no CUDA runtime. | I installed CUDA-enabled PyTorch in my project `.venv` and confirmed CUDA availability. |
| 4 GB mobile GPU memory | My GPU memory limits grid size, frame buffering, and concurrent visualizations. | I use small smoke settings, reduce resolution/recorded frames, close competing GPU workloads, and retain CPU fallback. |
| Sandbox vs host device access | My managed coding host may not receive the same GPU device permissions as my Linux terminal. | I use my Linux terminal output as host CUDA evidence and run device diagnostics in the environment intended for execution. |
| No standalone shell scripts | I have documented commands and VS Code launchers, but have not packaged shell automation. | I plan to add reproducible scripts and a run manifest. |
| Generic lower-bound requirements | My version ranges alone can resolve differently later and do not capture the CUDA wheel source. | I will create a lockfile/environment record after deciding my supported Python/PyTorch matrix. |
| Flow demos are not flight dynamics | My demos lack vehicle forces/torques, rigid-body states, attitude controller, motor mixer, propeller/ESC response, and sensor models. | I will retain them as visualization/numerical experiments and begin a distinct multirotor dynamics and control simulation. |
| Hardware status unknown | My attached brief names a proposed parts architecture, but I have no purchase/connection/test record in this workspace. | I will create an inventory and interface sheet, then record bench and calibration evidence as hardware becomes available. |

## Readiness summary

| Area | Current assessment |
| --- | --- |
| Linux/Python workflow | I demonstrated a venv, project dependencies, VS Code workflow, and command-line simulations. |
| CUDA host path | I demonstrated CUDA availability on my GTX 1650 in the Linux terminal. |
| Visualization / telemetry | I generated GIF/PNG/PT and JSON outputs with numerical summaries in logs. |
| Quadrotor plant/controller | I have only an illustrative one-axis pitch model; I have not implemented attitude/rate PID, a 6-DoF plant, motor mixer, or estimation stack. |
| Flight hardware / flight test | I have not documented physical integration, props-off checks, HIL, restrained tests, or free-flight results. |

# 7. Future implementation plan: simulation toward hardware

My next step is to build a model and test harness that answer flight-control questions directly. I will progress through gates with measurable acceptance evidence and will not treat a successful animation as a flight-readiness result.

| Stage | Implementation on Linux | Exit evidence / gate |
| --- | --- | --- |
| 0. Requirements and interfaces | I will choose the H743 board/firmware path, frame geometry, motor/ESC/propeller candidates, battery, sensor rates, control loop rates, modes, and safety behaviors. I will set angle/rate limits, allowed attitude error, settling/overshoot targets, latency budget, and logging fields. | I will complete this stage with versioned requirements, wiring/interfaces, an exact parts inventory, and a test matrix that distinguishes measured parameters from assumptions. |
| 1. Parameter identification | I will measure mass, center of gravity, arm lengths, inertia approximation/pendulum response, motor/prop thrust/torque curve, voltage sag, ESC command-to-thrust delay, and actuator limits. I will record IMU bias/noise and sample timing. | I will save calibration files with units, uncertainty, equipment/method, repeated measurements, and a plot/report. |
| 2. Multirotor simulation | I will add a nonlinear 6-DoF rigid-body model; body/world frame transforms; gravity; thrust and reaction torque; motor/ESC first-order dynamics; saturation, dead zones, battery effects; and optional ground/wind disturbances. | I will require bounded hover and commanded-attitude responses, documented equations and coordinate conventions, and time-step/solver convergence under refinement. |
| 3. Controller and estimator | I will implement rate PID first, then an attitude outer loop, mixer, anti-windup, filtering, setpoint shaping, arming/mode state machine, sensor-bias/noise, and estimator delays. I will mirror the intended H743 implementation as closely as feasible. | I will require step, disturbance, saturation, and sensor-fault tests to meet quantitative requirements across varied parameters, with error, rates, motor outputs, and timing logged. |
| 4. Automated evaluation | I will run deterministic sweeps over mass/CG, inertia, thrust, battery, delay, gains, wind, sensor noise, and failure cases. I will export CSV/JSON summaries and plots, tracking software revision and configuration hash. | I will require repeatable reports and a stability/robustness envelope, with no hidden parameter tuning between comparable runs. |
| 5. SITL and firmware path | I will select a flight stack deliberately (for example PX4, if it fits the H743 and my project goals) and use its supported Linux SITL path. I will keep my custom Python plant as an analysis tool or connect it only through a documented interface after protocol/clock behavior is clear. | I will verify firmware builds, simulator topics/protocols, control modes, and failsafe scenarios, and record exact firmware, simulator, and version. |
| 6. Props-off bench / HIL | With motor power safely isolated at first, I will verify board boot, sensor orientation/calibration, receiver input, arming interlocks, telemetry, motor order/direction, output limits, watchdog, link loss, low battery, and emergency disarm. I will progress to supported HIL with propellers removed or actuators isolated where appropriate. | I will keep a signed checklist and timestamped logs, verify control outputs and failsafes before fitting propellers, and follow the selected flight stack and hardware manufacturer procedures. |
| 7. Controlled flight validation | Only after the earlier gates pass, I will test one aircraft in a controlled, legally permitted environment using conservative limits and an independent pilot override. I will then test sensor additions and repeatability, and add second-aircraft coordination only after each aircraft passes independently. | I will use incremental flight cards for restrained/low-risk hover, short attitude maneuvers, failsafe rehearsal, repeatability, then dual-vehicle link-loss and mission-level behavior. |

## 7.1 Linux workflow optimizations

I will use a project `.venv` and a recorded dependency lock or constraints file. I will capture `python --version`, `pip freeze`, `torch.__version__`, `torch.version.cuda`, driver version, GPU name, git revision, command-line arguments, and run seed in a JSON run manifest.

I will keep `work/` for generated artifacts and organize them by experiment/run identifier. I will store source/configuration with results and preserve a baseline run.

I will add consistent CLI controls for resolution, seed, device (`auto`, `cuda`, `cpu`), animation frame rate, and output directory, plus a small batch mode for parameter sweeps.

I will use the GTX 1650 for accelerated tensor sweeps where the workload benefits. I will benchmark against CPU for realistic grid sizes and optimize memory/frame recording before increasing grid size.

I will use VS Code tasks or launch configurations to make common runs repeatable, ensuring the selected interpreter points at `.venv`. I will add checks around clear mathematical and I/O contracts.

For companion Pi work, I will separate the flight-controller interface, MAVLink/telemetry, mission logic, logging, and inter-aircraft transport. I will treat stale messages and link loss as explicit states with bounded timeouts.

## 7.2 Practical software boundaries

I will use Linux throughout my development pipeline for simulation, parameter fitting, plotting, version control, build automation, SITL, log review, and mission-level companion software. I will keep high-rate stabilization on the H743 MCU unless I later adopt and measure a separately justified real-time design; I will not assume that a general-purpose Raspberry Pi/Linux scheduler provides a hard-real-time inner loop.

# 8. Conclusion

I have moved beyond a basic CUDA probe: I can run Linux CUDA workloads, three flow-oriented simulations, generate visual artifacts, produce numerical/JSON checks, render a pitch-axis state-space illustration, use a runtime notebook, and launch common runs from VS Code. My clearest current engineering result is a working CUDA PyTorch environment on my GTX 1650 and repeatable execution of the visualization scripts.

I am at the simulation-tooling and concept-demonstration stage. My next substantial step is to create a separate, measurable multirotor dynamics/control stack grounded in my actual frame and propulsion hardware. I will progress through parameter identification, 6-DoF simulation, estimator/controller tests, and SITL, then props-off bench and HIL checks, to build a credible bridge toward my dual-prototype hardware-testing phase.

# References and evidence

[1] My project source and recorded runs. My workspace files: README.md, requirements.txt, .vscode/launch.json, Python sources and work/*.log / work/bfs_quick_report.json. Run values reproduce my captured local terminal/log evidence and were not independently rerun for this report.

[2] My dual-prototype design brief. My “Dual_Prototype_Quadcopter_Build_Schematic(1)(2).docx”; I use it as a project design brief describing my target architecture and staged objectives, not as proof of hardware implementation.

[3] PyTorch installation selector. Official source for choosing a PyTorch package matching the local platform and CUDA needs.

[4] PX4 Simulation. Official PX4 simulation overview; included as a candidate future workflow, not evidence that PX4 is currently installed or selected.

[5] PX4 Hardware Simulation. Official PX4 hardware simulation overview, including supported simulation/HIL concepts.

[6] PX4 HITL. Official PX4 HITL guide. The guide notes support limitations; verify the current recommended path for the selected firmware release before adopting it.

[7] PX4 Failsafe Simulation. Official PX4 failsafe simulation guide; use for scenario planning after selecting and versioning a flight stack.

PyTorch local install selector

PX4 Simulation

PX4 Hardware Simulation

PX4 HITL

PX4 Failsafe Simulation